Material Safety Data Sheet

Section I - Identification

Product Name: NanoVest Universal Investment Powder

Grade Name: Quartz and Feldspar

Manufacturer:  WHW Plastics Limited

Therm Road

Hull

East Yorkshire, UK

0044 (0)1482 329 154

Issue Date 10/22/09

Revision Date

Section II - Composition Information

| Substance | C.A.S. | OSHA PEL (mg/m3) | ACGIH TLV (mg/m3) |
| --- | --- | --- | --- |
| Quartz | 14808-60-7 | 0.1 | 0.1 |
| Feldspar | 1332-58-7 | 0.1 | 0.1 |
| Binders | Not established | * | * |
| *This material is a Non-Hazardous Substance |  |  |  |

Section III - Health Hazard Data

Effects of Overexposure

Eyes: Particles may cause mild irritation

Skin: May dry skin over prolonged exposure

Inhalation: Large amounts of dust may cause nuisance conditions-coughing, sneezing and nasal irritation

Ingestion: Hardens if wetted and may cause obstruction

Emergency and First Aid Procedures

Eyes: Flush eyes with water for 15 minutes – Call Physician if irritation continues

Skin: Wash contaminated area with water

Inhalation: Remove to fresh air

Ingestion: See physician.

Section IV - Fire and Explosion Data

Flash Point: None

Extinguishing Media: Not combustible

Special Fire Fighting Procedures: None

Unusual Fire & Explosion Hazards: None

Section V - Spill or Leak Procedures

Spill or Leak Procedures: Sweep or vacuum material into waste container for disposal. Avoid dusting conditions.

If washed down, may plug drains.

Waste Disposal: Disposal in accordance with Local, State and Federal Regulations.

Section VI - Special Precautions

Maintain good ventilation to avoid exceeding TLV levels.

Practice good housekeeping by not allowing dust to collect on ledges, floors, machinery or equipment.

Section VII - Special Protection

Respiratory: Provide general ventilation and use an NIOSH approved mask respirator.

Protective Equipment: Goggles may be needed to avoid splash irritation to the eyes.  Gloves may be desirable in specific work situations

Section VIII - Physical Data

Boiling Point: No data

Melting Point: 5600º F

Specific Gravity: 2.45

Appearance & Odor: Powder - odorless

Solubility in Water: 0.2%

Section IX - Reactivity Data

Stability: Stable

Incompatibility: Acids

Hazardous Decomposition Products: Above 2300º F

Polymerization: WILL not occur